Communication problem solving

Table of Contents

1. RAPID code	2

1.1 Target Setting	2

1.2Discard code	4

2. problem solvin	7

2.1 setup	7

2.2 Do not use ASCII error reporting	9

2.3An error using ASCII without a trailing number is reported	10

2.4 Byte count too high	13

3. Link	13

1. RAPID code

1.1 Target SettingMODULE Module1

VAR socketdev server; 
    VAR socketdev client; 
    VAR string signalData; 
    VAR rawbytes data; 
    VAR robtarget currentPos;

PROC main() 
        SocketCreate server; 
        SocketBind server, "127.0.0.1", 5000; 
        SocketListen server; 
        SocketAccept server, client;

WHILE TRUE DO 

           currentPos := CRobT(\Tool:=MyNewTool);

            SocketReceive client, \RawData:=data; 
            UnpackRawBytes data,1,signalData,\ASCII:=2;

IF signalData = "X_MOVE,100,0,0" THEN 
                currentPos.trans.x := currentPos.trans.x + 10;
            ELSEIF signalData = "X_MOVE,-100,0,0" THEN 
                currentPos.trans.x := currentPos.trans.x - 10;
            ELSEIF signalData = "Y_MOVE,0,100,0" THEN 
                currentPos.trans.y := currentPos.trans.y + 10;
            ELSEIF signalData = "Y_MOVE,0,-100,0" THEN 
                currentPos.trans.y := currentPos.trans.y - 10;
            ELSEIF signalData = "Z_MOVE,0,0,100" THEN 
                currentPos.trans.z := currentPos.trans.z + 10;
            ELSEIF signalData = "Z_MOVE,0,0,-100" THEN 
                currentPos.trans.z := currentPos.trans.z - 10;
            ENDIF

MoveL currentPos, v100, z10, tool0;
        ENDWHILE

SocketClose client; 
        SocketClose server; 
    ENDPROC 
ENDMODULE

1.2Discard code

The yellow part continues to report an error, indicating that the configuration fails and debugging succeeds

RAPID1:

MODULE Module1

!***********************************************************

!

! Module:  Module1

!

! Description:

!   <Insert description here>

!

! Author: Jiajun Guo

!

! Version: 1.0

!

!***********************************************************

!***********************************************************

!

! Procedure main

!

!   This is the entry point of your program

!

!***********************************************************

!PROC main()

!Add your code here

!ENDPROC

!VAR socketdev MySocket;

!VAR string signalData;

!MODULE GestureControl

VAR iodev MySocket;

VAR robtarget pStart := [[500,0,1000],[1,0,0,0],[1,0,0,0],[0,0,0,0]];

VAR robtarget pXMove := [[600,0,1000],[1,0,0,0],[1,0,0,0],[0,0,0,0]];

VAR robtarget pYMove := [[500,100,1000],[1,0,0,0],[1,0,0,0],[0,0,0,0]];

VAR robtarget pZMove := [[500,0,1100],[1,0,0,0],[1,0,0,0],[0,0,0,0]];

PROC main()

VAR string signalData;

! TCP/IP

!Open MySocket \IP:="127.0.0.1", \Port:=5000;

!Open MySocket \IP:="127.0.0.1", \Port:=5000, \IODevice:="MySocket";

Open MySocket \IP:="127.0.0.1", \Port:=5000, \IODevice:="TCPIP1";

WHILE TRUE DO

Read MySocket, signalData;

IF signalData = "START" THEN

MoveL pStart, v100, z10, tool0;

ELSEIF signalData = "STOP" THEN

StopMove;

ELSEIF signalData = "X_MOVE" THEN

MoveL pXMove, v100, z10, tool0;

ELSEIF signalData = "Y_MOVE" THEN

MoveL pYMove, v100, z10, tool0;

ELSEIF signalData = "Z_MOVE" THEN

MoveL pZMove, v100, z10, tool0;

ENDIF

ENDWHILE

Close MySocket;

ENDPROC

ENDMODULE

2. problem solvin

2.1 setup

Change Options

Controller tab > Configuration > Communication > Transmission Protocol

2.2 Do not use ASCII error reporting

UnpackRawBytes data, 1, signalData;

2.3An error using ASCII without a trailing number is reported

UnpackRawBytes data, signalData \ASCII;

Refer to page 509 of this document

2.4 Byte count too high

UnpackRawBytes data,1,signalData,\ASCII:=15;

3. Link

Document Links：
https://search.abb.com/library/Download.aspx?DocumentID=3HAC050917-001&LanguageCode=en&DocumentPartId=&Action=Launch

YouTube link：
https://www.youtube.com/watch?v=JGJt17c69Bk